ROSS NULL SOFTWARE

------------------

The following article appeared in Telescope Making magazine

describing the operation of the Ross Null software.

Ross Null Setup Software	Doug George

The Ross Null Test, described in detail by Peter Ceravolo in TM�#39, provides a much more precise and convenient way to test a telescope mirror.  By making an ideal mirror appear nearly spherical to the tester, the tedious measurements of the focault test are eliminated.  Errors in the mirror surface are immediately obvious since any shadows indicate departures from the ideal figure.

Setting up the test is quite simple once you know the required spacing between the mirror and lens.  In Peter's article, a series of graphs are used to determine the correct spacing.  Obviously, this is not the most convenient method.  Following Peter's suggestions, I have written a computer program which allows rapid, effortless calculation of the required spacings.

Program Operation

Upon starting the program, you will be prompted for the mirror diameter, focal length, and conic constant (enter 0�for a parabola).  Measurement in inches must be used where appropriate.  Next, you will be asked for the lens' surface radius on the image side, radius on the mirror side, thickness, and refractive index.  If you are using the usual plano-convex lens, the radius on the image side should be infinite (flat), so you should enter a large number such as 9e99.  The mirror-side radius should be negative for the plano-convex lens.  Finally, you will be asked for the distance from the lens to mirror.  If you want the program to determine this number, enter zero.  Otherwise, you can enter a value, and the program will determine the resulting spherical aberration.  This is useful in checking the sensitivity of your setup to positioning errors.

Once this information is entered, the program will output intermediate results as it iterates towards the solution.  This program does not include any error checking (other than the compiler's).  The intermediate results help you keep an eye on the program operation.

When the computer has iterated the error down to a reasonable value, it will output a summary.  The summary includes the object to lens distance, lens to mirror distance (measured at the center of curvature), the working diameter of the lens, the residual spherical aberration at the 70% zone and mirror edge, and the maximum optical path difference (in waves).  The working diameter of the lens is important:  if the marginal rays are too close to the edge of the lens, the results will be suspect.  For commercially-purchased lenses, it is probably best to stay within half the diameter.  The residual spherical aberration is measured in inches.  The quarter-wave tolerance indicates the maximum amount of spherical aberration for a quarter-wave error.  If measurements made with the knife-edge micrometer exceed this limit, then the mirror does not meet 1/4 wave on the wavefront.  Finally, the maximum OPD gives the residual spherical aberration in waves.  This must be significantly smaller than the tolerance you plan to polish your mirror to.

Note that this program works for non-parabolic mirrors.  It also does not require the null lens to be plano-convex.  Of course, I cannot guarantee that the results will converge for an arbitrary combination of optics - after all, there might not be a solution!

Program Description

As a firm believer that BASIC is completely obsolete, I wrote the program using Turbo Pascal.  The accompanying program listings are simplified to use only standard Pascal so it will work with almost any Pascal compiler.  The software is also available on disk for both the PC and MacIntosh (see below).  Both source and executable programs are provided.  The PC version includes a few extra features such as an optional graphics display.  The program takes about 10 seconds to run on a 25 MHz 386 computer (even faster with a math coprocessor).

The program calculates the lens spacing by performing a ray trace of the optics.  It guesses at a starting position for the lens relative to the mirror, and adjusts that position until the spherical aberration is minimized.

The program consists of several subroutines for clarity.  The ArcSin and Power routines perform some simple math functions which the compiler does not provide.

The AsphericReflectRay, TraceRay, and Transfer routines perform the ray tracing mathematics.  AsphericReflectRay determines the point at which the light ray intersects the mirror.  An eight-order approximation to the aspheric is used.  The intersection point is determined iteratively using a gradient method.  Once the intersection point is determined, simple geometry determines the reflection path.  TraceRay calculates the deflection of a light ray as it passes through a lens surface.  Transfer carries the light ray from one surface to the next.

CalculateLi uses the ray tracing routines to trace the path of a single light ray through the optics.  Given positions of the lens and mirror, it determines the resulting distance between between the lens and the image.  With spherical aberration, the height of the ray affects the focus point.  Therefore, the height of the ray striking the first surface of the lens is also input.  The routine calculates the ray height at the other optical surfaces along the way.

CalculateLo determines the correct place to put the object (light source) so that the image falls in the same place.  This is done iteratively by moving the object slightly towards the image, and determining the new image point using CalculateLi.  This process is repeated until the error is sufficiently small.

CalculateSA takes as input the guess at the correct distances between the mirror, lens, and object (light source).  It also needs the mirror semi-diameter, and a guess at the height of a marginal ray at the first surface of the lens.  It uses CalculateLo to determine the object spacing, and then adjusts the ray height to keep the light ray intersecting the mirror edge.  CalculateLo is then called again to tweak up the object distance.  CalculateLi is then used to determine the image distance for a ray at 10% of the marginal height.  The difference in the image positions for these two rays gives the spherical aberration from center to edge.

The CalculateSA routine is used iteratively by the main program to determine the correct lens spacing.  After propting the user for the required input, initial guesses are made for the spacings and marginal ray height at the lens.  Another parameter, AdjustGain, is calculated based on the focal ratio.  For a given spherical aberration, this increases the position adjustment for a long focus mirror, and reduces it for a fast mirror.  This ensures rapid convergence on the solution without the mathematics going unstable.

The main loop calculates the spherical aberration given the present guessed parameters.  It multiplies the resulting spherical aberration by the AdjustGain factor to determine the amount to adjust the mirror to lens spacing.  Each time the spacing is moved, it becomes closer and closer to the optimum value.  The loop exits when the residual aberration is small enough.

With the marginal spherical aberration nearly at zero, the residual error in the test will be the error at the 70% zone.  The program calculates this and outputs the result summary.

Program Testing

If you typed in the listing accompanying this article, the program should be tested to ensure it is working properly.  The accompanying example shows the program input and output for a sample optical setup.  Enter the same data and verify that the results are similar.  Note that small differences in mathematical precision from with different compilers and computer types may result in slightly different numerical results.

Summary

This program greatly simplifies the use of the Ross Null test.  The optical setup can be determined in a matter of seconds, and the probability of error is reduced.  The sensitivity of the test to the placement of the mirror and lens can be determined by manually entering values to either side of the optimum positioning.

Be careful and double check your results.  The old adage, "garbage in, garbage out" certainly applies here.  When using the test for the first time, it might be a good idea to test a mirror of known quality.  Any mistakes in the setup or problems with the null lens will be immediately obvious.  When figuring a mirror, occasionally double-check the results with a focault test or other method - you don't want to do a Hubble!

I would like to thank Peter Ceravolo for his valuable assistance and encouragement.

EXAMPLE

Ross Null Test Setup Calculations V1.1

Copyright (c) 1990, Doug George

66 Shetland Way, Kanata, Ontario, Canada, K2M 1S4

This shareware program may be freely distributed and used

All Dimensions in Inches

Mirror Parameters

Diameter                       ->   12.500000

Focal Length                   ->   50.000000

Conic Const. = 1 - Eccent. ** 2

Parabola = 0, Sphere = 1

Conic Constant                 ->    0.000000

Lens Parameters

Sign Convention:

With mirror on right, ) = -ve, ( = +ve

Image Side Radius              ->    Infinite

Mirror Side Radius             ->  -10.070000

Thickness                      ->    0.442000

Diameter                       ->    3.000000

Refractive Index               ->    1.514323

Enter Lens To Mirror Distance

Or Enter 0 to Calculate        ->    0.000000

Object-Lens =    9.504887  Lens-Mirror =   80.420864  Marginal SA =   -0.014379

Object-Lens =    9.684674  Lens-Mirror =   79.670118  Marginal SA =   -0.005493

Object-Lens =    9.751542  Lens-Mirror =   79.383331  Marginal SA =   -0.002344

Object-Lens =    9.779793  Lens-Mirror =   79.260940  Marginal SA =   -0.000975

Object-Lens =    9.791481  Lens-Mirror =   79.210018  Marginal SA =   -0.000391

Object-Lens =    9.796189  Lens-Mirror =   79.189605  Marginal SA =   -0.000154

Object-Lens =    9.798022  Lens-Mirror =   79.181584  Marginal SA =   -0.000060

Results:

Object to Lens Distance         =    9.798022

Lens to Mirror Distance         =   79.181584

Working Diameter of Lens        =    2.621250

70% Zone Spherical Aberration   =   -0.000518

Marginal Spherical Aberration   =   -0.000058

Quarter-wave Tolerence          =    0.004614

Maximum Optical Path Difference =    0.028046

Enter Lens To Mirror Distance

Or Enter 0 to Calculate        ->    0.000000

Object-Lens =    9.606583  Lens-Mirror =   80.000000  Marginal SA =   -0.009537

Object-Lens =    9.750075  Lens-Mirror =   79.389656  Marginal SA =   -0.002396

Object-Lens =    9.785443  Lens-Mirror =   79.236339  Marginal SA =   -0.000703

Object-Lens =    9.795768  Lens-Mirror =   79.191352  Marginal SA =   -0.000178

Results:

Object to Lens Distance         =    9.795768

Lens to Mirror Distance         =   79.179936

Working Diameter of Lens        =    2.619898

70% Zone Spherical Aberration   =   -0.000521

Marginal Spherical Aberration   =   -0.000066

Quarter-wave Tolerence          =    0.004616

Maximum Optical Path Difference =    0.028204

Additional Notes on the PC version of RossNull:

-----------------------------------------------

Grahpical Display

-----------------

To see a graphical display of the calculations, enter:

C>rossnull /g

This will have the effect of slowing down the calculations

substantially, but you will be able to see what is happening.

An EGA or VGA display is required.  Hercules cards may also work.

Input from File

---------------

To use a file for input, instead of typing in values, enter:

C>rossnull <filename>

where <filename> is replaced by the name of the file you want to use.  An example is supplied on the disk called ROSSNULL.DAT.

A typical input file is:

12.5

50

0

9e99

-10.07

.442

3

1.514323

0

These numbers represent the following:

mirror diameter

mirror focal length

mirror conic constant

lens object-side focal length

lens mirror-side focal length

lens refractive index

lens diameter

lens refractive index

zero to request automatic iteration

Output to File

--------------

To use a file for input and output, enter:

C>rossnull <input filename> <output filename>

Additional Comments

-------------------

This program uses an iterative approach to determine the correct lens positioning.  This iterative approach uses a "gain" setting to determine how much to move the lens on each step.  This gain setting is determined from the mirror and lens f/ratio, using an empirically-determined formula.  If the program fails to converge, try increasing the lens diameter.  This has no effect other than reducing the "gain" setting.

The starting position for the lens is determined from the lens and mirror focal length.  The program assumes that the mirror focal length is longer than that of the lens.

The program performs little error checking and assumes that the input is reasonable.  Illegal input (such as a negative FL lens) may cause the program to crash or not terminate.   The program should operate correctly for all reasonable input values, although I obviously could not test all possible configurations since there are an infinite number of them!